ITU - Telecommunication Standardization Sector	Temporary Document 34 (PLEN)

STUDY GROUP 16

Geneva, 5 – 15 February 2002

Question:		ALL/16

SOURCE*:	Editor

TITLE:	H.248 Annex L Amendment 1 for Approval

___________________

Introduction

This contribution contains H.248 Annex L (07/2001) Amendment 1 for Approval

itu-t recommendation H.248 Annex L amendment 1

Summary

This Amendment enhances Recommendation H.248 Annex L (Error Code and Service Change Reason Description, 07/2001).

The modifications are underlined and marked with revision bars.

Amendment to Recommendation H.248 Annex L

This Amendment enhances Recommendation H.248 Annex L (Error Code and Service Change Reason Description, 07/2001). These enhancements includes new codes and service change reasons introduced by H.248 (Gateway Control Protocol, 02/2002).

The modifications are underlined and marked with revision bars.

- To Clause L.4.2.27 Error Code # 451

#### L.4.2.27 Error Code # : 451 Name: No such event in this package

Definition: The command including the Event Name is not executed because  it is not considered to be a part of this version of package.

Package: -

Reference: -

Error Text in the error Descriptor: The Event Name is included in the Error text in the error descriptor

Comment: -

- To Clause L.4.2.30 Error Code # 454

#### L.4.2.30 Error Code # : 454 Name: No such parameter value in this package

Definition: The command including the parameter value is not executed because it is not considered  be a part of this version of package.

Package: -

Reference: -

Error Text in the error Descriptor: The Parameter Name is included in the Error text in the error descriptor.

Comment: -

- To Clause L.4.2.x Error Code # 506

#### L.4.2.xx Error Code # : 506 Name: Number of TransactionPendings Exceeded

Definition: Indicates that the maximum number of TransactionPendings have been received and that the transaction can be assumed to be in error.

Package: -

Reference: -

Error Text in the error Descriptor: -

Comment: -

- To Clause L.2.2.x Reason # : xxx

#### L.5.2.17 Reason # : 916 Name: Packages Change

Definition: This indicates that the packages or package versions implemented on the Media Gateway have changed. The MGC should consider performing an AuditValue of the Media Gateway to determine which packages have changed.

Reference: -

Text in the Service Change extension: -

Comment: -

#### L.5.2.18 Reason # : 917 Name: Capabilities Change

Definition: This indicates that the capabilities of the Media Gateway have changed. This is used in the case where the package has not changed but the capabilities of properties/parameters have. The MGC should consider performing an AuditCapabilities of the Media Gateway to determine which capabilities have changed.

Reference: -

Text in the Service Change extension: -

Comment: -