|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – D 118 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | (WP 2/16) |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 3/16 | Geneva, 26 July - 5 August 2005 |  |  |
| STUDY GROUP 16 – DELAYED CONTRIBUTION 118 |  |  |  |  |  |
| Source: |  | LM Ericsson |  |  |  |
| Title: |  | H.248.1v3: Clarifications and Corrections |  |  |  |

# 1	Introduction

This document proposes a number of changes to H.248.1v3.

# 2	Discussion

This contribution proposes a number of changes to H.248.1v3 which are described below. Some of the changes were identified at the time of H.248.1v3 editing and have been included. These have indicated below.

To be included in H.248.1v3

- H.248.1: RequestID and StreamID for IndAuditSignals

- H.248.1: Incorrect References

- H.248.1: IANA Lists

- H.248.1: Order of Command Replies

- H.248.1: Profile Template Clarification

- H.248.1: Non-backwards compatible ABNF change

Already Included in H.248.1 v3 editors input:

- H.248.1: Optional RequestID in IndAudEventsDescriptor

- H.248.1: Auditing SignalList by ID only

- H.248.1: Value of Statistics at a Termination Level

- H.248.1: Statistics and multiple streams per media descriptor

- H.248.1: ABNF CHOOSE and ALL clarification

The discussion for each of the above items is included below.

# 3	Proposal

## 3.1	H.248.1: RequestId and StreamID for IndAuditSignals (Partly Included)

| Description: | The capability to distinguish between Notify Completions for different signals was added to H.248.1v3. This took the form of a RequestID parameter to be associated with the individual signal. This parameter is missing from the Individual Audit of Signals unlike the RequestID that is associated with events. Furthermore comparing the ASN.1 and ABNF structures it has been found that the ASN.1 supports the ability to provide a StreamID associated with the signal however the ABNF does not. Both the ASN.1 and the ABNF support the StreamID being associated with Events. The text of H.248.1 section 7.2.5 AuditValue and 7.2.6 AuditCapability do not mention the ability to provide a StreamID for auditing Signals. However H.248.1 section 7.1.11 allows an optional StreamID parameter. Therefore it is proposed that an optional StreamID parameter is included in the ABNF and this be reflected in section 7.2.5 and 7.2.6 |
| --- | --- |
| Reference: | This contribution. |

[Begin Correction]

### 7.2.5	AuditValue

…

•	To audit a signal, the relevant signal list ID and/or signal ID are provided. The StreamID and/or RequestID are optional. Only if the signal is active are the values of all the signal parameters returned including: the KeepActive indication, signal type, duration, signal completion indication and package defined properties.

…

### 7.2.6	AuditCapability

…

•	To audit a signal the relevant signal list ID and/or signal ID are provided. The StreamID and/or RequestID are optional. A list of possible values associated with each signal parameter is returned (including: the keepactive indication, signal type, duration, signal completion indication and package defined properties).

…

### A.2	ASN.1 Specification

IndAudSignal ::= SEQUENCE

{

signalName				PkgdName,

streamID				StreamID OPTIONAL,

...,

requestID				RequestID OPTIONAL

}

### B.2	ABNF Specification

indAudsignalRequest	= signalName [COMMA sigStream] [COMMA sigRequestID]

[End Correction]

## 3.2	H.248.1: Incorrect References (Not Included in editor’s draft)

| Description: | There is an incorrect reference to B.3 when referring to the coding of sub-lists. The reference should indicate B.2. |
| --- | --- |
| Reference: | Subject: [Megaco] Ambiguities,Errors and forgotten things in Packages Desc ription Date: Thu, 9 Jun 2005 23:12:40 +1000 From: Contardi Angelo <Angelo.Contardi@italtel.it> |

[Begin Correction]

### 12.1.2	Properties, 12.1.5 Statistics, 12.2 Guidelines to defining parameters to events and signals

Sub-list: a list of several values from a list. The type of sub-list SHALL also be specified. The type shall be chosen from the types specified in this section (with the exception of sub-list). For example, Type: sub-list of enumeration. The encoding of sub-lists is specified in Annexes A and B.2.

[End Correction]

## 3.3	H.248.1: IANA Lists (Not Included in editor’s draft)

| Description: | In H.248.1 section 12.3 it mentions that lists may be registered with IANA. “Type:List” is not described by H.248 but “Type: Sub-List of” is described. Currently there are no lists registered with IANA nor is their any mechanism defined to do so. Thus it is proposed to remove this section. The removal of this section will not cause any backwards compatibility issue as it is not seen in the H.248 encoding and the mechanism has never been used. |
| --- | --- |
| Reference: | Subject: [Megaco] Ambiguities,Errors and forgotten things in Packages Desc ription Date: Thu, 9 Jun 2005 23:12:40 +1000 From: Contardi Angelo <Angelo.Contardi@italtel.it> |

[Begin Correction]

[End Correction]

## 3.4	H.248.1: Order of Command Replies (Not Included in editor’s draft)

| Description: | In H.248.1 section 8 states that commands are executed sequentially in the order they are in a transaction. Ie. Transactions guarantee ordered Command processing. That is, Commands within a Transaction are executed sequentially. Ordering of Transactions is NOT guaranteed; transactions may be executed in any order, or simultaneously; however, transaction replies should be executed before transaction requests when both are contained in a message. However no such statement exists for the command reply. If one executes the commands sequentially then it can be assumed that the command replies be in the order that they were received. If replies are sent back in a different order this causes problems in that the whole transaction must be first processed before it can be determined what replies match the original requests. Therefore it is proposed to indicate that command replies shall be in the same order as command requests. |
| --- | --- |
| Reference: | This contribution |

[Begin Correction]

### 8	Transactions

…

A TransactionReply includes the results for all of the Commands in the corresponding TransactionRequest. The TransactionReply includes the return values for the Commands that were executed successfully, and the Command and error descriptor for any Command that failed. The Commands replies are returned in the Transaction reply in the order they were received in the corresponding TransactionRequest. TransactionPending is used to periodically notify the receiver that a Transaction has not completed yet, but is actively being processed.

…

[End Correction]

## 3.5	Profile Template Clarification (Not Included in Editor’s draft)

| Description: | The current H.248 Profile template text may be confusing as to what needs to be specified for events and signals descriptors. |
| --- | --- |
| Reference: | This contribution |

[Begin Correction]

5.7.2	Events Descriptor

All events contained in this profile may be set on any termination/stream [with the following exceptions].

Note: The text in [] is optional and only included if there are such exceptions.

5.7.4	Signals Descriptor

All signals contained in this profile may be set on any termination/stream [with the following exceptions].

Note: The text in [] is optional and only included if there are such exceptions.

[End Correction]

## 3.6	H.248.1: Optional RequestID in IndAudEventsDescriptor (Included in Editor’s draft)

| Description: | H.248.1 sections 7.2.5 AuditValue and 7.2.6 AuditCapability indicate that it is possible to Audit Events by including an optional RequestId. Ie. • To audit an event the relevant stream id (optional), eventID, requestID (optional) are provided. A list of possible values associated with each event parameter is returned (including: event actions and packaged defined parameters). In the ASN.1 of H.248.1 this is possible however in the ABNF the RequestID is manadatory. This should be made optional. |
| --- | --- |
| Reference: | Subject: Re: [Megaco] Individual audit: differences in text & binary Date: Mon, 16 May 2005 11:42:47 +1000 From: Christian Groves <christian.groves@ericsson.com> To: Frank Reno <hmegaco@yahoo.com> |

[Begin Correction]

### B.2	ABNF Specification

indAudeventsDescriptor	= EventsToken [EQUAL RequestID] LBRKT

indAudrequestedEvent RBRKT

[End Correction]

## 3.7	Non-backwards compatible ABNF change

| Description: | The possibility to define context properties via packages was added in H.248.1v3. However when this was added to the ABNF it deleted some existing ABNF constructs. The constructs would be visible in protocol messages. This constitutes a non-backwards compatible change. The H.248.1v2 should be re-instated. Note: Yellow highlighting indicates structure that needs to be changed. |
| --- | --- |
| Reference: | This contribution |

[Begin Correction]

5.7.2 Events Descriptor

; EmergencyOff to be used in MG to MGC direction only in H.248.1 V1 and V2

; EmergencyToken or EmergencyOffToken, but not both

contextProperty		= (topologyDescriptor / priority /

EmergencyToken / EmergencyOffToken

/ iepsValue / contextAttrDescriptor)

contextAudit		= ContextAuditToken LBRKT

(contextAuditProperties *(COMMA

contextAuditProperties))

/ indAudcontextAttrDescriptor)

RBRKT

; at-most-once

contextAuditProperties	= (TopologyToken / EmergencyToken /

PriorityToken /

IEPSToken / pkgdName)

indAudcontextAttrDescriptor

= ContextAttrToken LBRKT contextAuditProperties

*(COMMA contextAuditProperties) RBRKT

contextAttrDescriptor	= ContextAttrToken LBRKT (contextIdList /

propertyParm *(COMMA propertyParm)) RBRKT

[End Correction]

## 3.8	H.248.1: Auditing SignalList by ID only (Included in editor’s draft)

| Description: | H.248.1 sections 7.2.5 AuditValue and 7.2.6 AuditCapability indicate that it is possible to Audit a Signals list by specifying the SignalListID only. The ASN.1 supports this capability. Ie. IndAudSeqSigList ::= SEQUENCE { id INTEGER(0..65535), signalList IndAudSignal OPTIONAL } However the ABNF makes it mandatory to include a signalID this should be changed. |
| --- | --- |
| Reference: | Subject: Re: [Megaco] Individual audit: differences in text & binary Date: Mon, 16 May 2005 11:42:47 +1000 From: Christian Groves <christian.groves@ericsson.com> To: Frank Reno <hmegaco@yahoo.com> |

[Begin Correction]

### B.2	ABNF Specification

indAudsignalList		= SignalListToken EQUAL signalListId [LBRKT

indAudsignalListParm RBRKT]

[End Correction]

## 3.9	H.248.1: Value of Statistics at Termination Level (Included)

| Description: | In H.248.1 v3 the capability to have Statistics on a stream or termination level was added. However it is unclear what the value of a Statistics on a Termination level is if there are multiple streams on the termination. To clarify this situation it is proposed that the value of a Statistic on a termination level is the sum of the values if it had been placed on all the streams in the termination. |
| --- | --- |
| Reference: | This contribution |

[Begin Correction]

7.1.15	Statistics Descriptor

… termination/stream a Statistics Descriptor with a single Statistic with the PackageID specified and the Statistic ID wildcarded with “all” is issued. The receipt of an empty descriptor means that no statistics shall be collected for a specific termination.

The value of a Statistic at a Termination level is the result of a meaningful superior function (like for instance sum or average)of the values as if it had been placed on all the streams in the termination. Such a superior function is dependent of the particular statistic type. Unless specified otherwise in the package that defines a particular statistic, the default behaviour is a sum of the values.

By default, termination and stream level statistics are reported when the Termination ceases to exist or is returned to the NULL context due to a Subtract command, unless an empty Statistic Descriptor had earlier been used to indicate that no statistics are to be collected. …

[End Correction]

## 3.10	Statistics and multiple streams per media descriptor (Included in Editor’s draft)

| Description: | In the ABNF encoding the indAudmediaDescriptor allows for only a single indAudstreamDescriptor. So to audit the statistics on multiple streams the MGC must supply multiple Media descriptors. However for the binary encoding the MGC indicate multiple streams per media descriptor. The ABNF should be updated to allow multiple streams per Media Descriptor. |
| --- | --- |
| Reference: | This contribution |

[Begin Correction]

B.2	ABNF specification

indAudmediaDescriptor	= MediaToken LBRKT indAudmediaParm * (COMMA

indAudmediaParm)RBRKT

; either streamParm or streamDescriptor but not both

indAudmediaParm		= indAudstreamParm / indAudstreamDescriptor /

indAudterminationStateDescriptor

indAudstreamParm		= (indAudlocalControlDescriptor /

indAudstatisticsDescriptor)

[End Correction]

## 3.11	ABNF CHOOSE and ALL clarification (Included in editor’s draft)

| Description: | It is described in the H.248.1v3 procedures that ALL is equal to * and CHOOSE is equal to $ however this is not stated in the ABNF. The ASN.1 does state how CHOOSE and ALL is achieved it is proposed to clarify this in the ABNF. |
| --- | --- |
| Reference: | Subject: R: R: [Megaco] Ambiguities,Errors and forgotten things in Package s D escription Date: Fri, 17 Jun 2005 20:12:57 +1000 From: Contardi Angelo <Angelo.Contardi@italtel.it> |

[Begin Correction]

B.2	ABNF specification

parmValue			= (EQUAL alternativeValue / INEQUAL VALUE)

; the (Safe)Char “$” means CHOOSE

; the (Safe)Char “*” means ALL

alternativeValue		= (VALUE

/ LSBRKT VALUE *(COMMA VALUE) RSBRKT

; sublist (i.e. A AND B AND ...)

/ LBRKT VALUE *(COMMA VALUE) RBRKT

; alternatives (i.e. A OR B OR ...)

/ LSBRKT VALUE COLON VALUE RSBRKT)

; range

[End Correction]

__________________